Use of dual element isotope analysis and microcosm studies to determine the origin and potential anaerobic biodegradation of dichloromethane in two multi-contaminated aquifers

Natàlia Blázquez-Pallía,b, Orfan Shouakar-Stashc,d,e, Jordi Palauf,g, Alba Trueba-Santisoa, Joan Variasb, Marçal Boschb, Albert Solerf, Teresa Vicenta, Ernest Marco-Urreaa,* and Mònica Rosellf

a Departament d’Enginyeria Química, Biològica i Ambiental, Universitat Autònoma de Barcelona (UAB), c/ de les Sitges s/n, 08193 Cerdanyola del Vallès, Spain.

b Litoclean, S.L., c/ Numància 36, 08029 Barcelona, Spain.

cDepartment of Earth and Environmental Sciences, University of Waterloo, Waterloo, Ontario N2L 3G1, Canada

dIsotope Tracer Technologies Inc., Waterloo, Ontario N2 V 1Z5, Canada

eSchool of Engineering, University of Guelph, Guelph, Ontario, N1G 2W2, Canada

fGrup MAiMA, SGR Mineralogia Aplicada, Geoquímica i Geomicrobiologia, Departament de Mineralogia, Petrologia i Geologia Aplicada, Facultat de Ciències de la Terra, Institut de Recerca de l’Aigua (IdRA), Universitat de Barcelona (UB), c/ Martí Franquès s/n, 08028 Barcelona, Spain.

gInstitute of Environmental Assessment and Water Research (IDAEA), CSIC, Jordi Girona 18-26, 08034 Barcelona, Spain

*Corresponding author:

Departament d’Enginyeria Química, Biològica i Ambiental, Universitat Autònoma de Barcelona (UAB), c/ de les Sitges s/n, 08193 Cerdanyola del Vallès, Spain. E-mail: ernest.marco@uab.cat, Phone: +34 935812694.

Abstract

Many aquifers around the world are impacted by toxic chlorinated methanes derived from industrial processes due to accidental spills. Frequently, these contaminants co-occur with chlorinated ethenes and/or chlorinated benzenes in groundwater, forming complex mixtures that become very difficult to remediate. In this study, a multi-method approach was used to provide lines of evidence of natural attenuation processes and potential setbacks in the implementation of bioremediation strategies in multi-contaminated aquifers. First, this study determined i) the carbon and chlorine isotopic compositions (δ13C, δ37Cl) of several commercial pure phase chlorinated compounds, and ii) the chlorine isotopic fractionation (εCl = -5.2 ± 0.6‰) and the dual C–Cl isotope correlation (ΛC/Cl = 5.9 ± 0.3) during dichloromethane (DCM) degradation by a Dehalobacterium-containing culture. Such data provide valuable information for practitioners to support the interpretation of stable isotope analyses derived from polluted sites. Second, the bioremediation potential of two industrial sites contaminated with a mixture of organic pollutants (mainly DCM, chloroform (CF), trichloroethene (TCE), and mono-chlorobenzene (MCB)) was evaluated. Hydrochemistry, dual (C–Cl) isotope analyses, laboratory microcosms, and microbiological data were used to investigate the origin, fate and biodegradation potential of chlorinated methanes. At Site 1, δ13C and δ37Cl compositions from field samples were consistent with laboratory microcosms, which showed complete degradation of CF, DCM and TCE, while MCB remained. Identification of Dehalobacter sp. in CF-enriched microcosms further supported the biodegradation capability of the aquifer to remediate chlorinated methanes. At Site 2, hydrochemistry and δ13C and δ37Cl compositions from field samples suggested little DCM, CF and TCE transformation; however, laboratory microcosms evidenced that their degradation was severely inhibited, probably by co-contamination. A dual C-Cl isotopic assessment using results from this study and reference values from the literature allowed to determine the extent of degradation and elucidated the origin of chlorinated methanes.

- Introduction

Dichloromethane (DCM) is a probable human carcinogen (IARC, 2016), included in the list of priority pollutants of both the U.S. Agency for Toxic Substances and Disease Registry (2016) and the European Commission (2012). DCM can be naturally released by oceanic sources, wetlands, volcanoes and macroalgae (Gribble, 2010), but its detection in groundwater is often a result of its extensive use, accidental spills and improper storage in chemical, pharmaceutical, and petroleum industrial facilities, among others (Marshall and Pottenger, 2016).

Numerous bacteria capable of using DCM as a growth substrate under aerobic conditions have been identified, including various strains of Hyphomicrobium (Heraty et al., 1999; Hermon et al., 2018; Nikolausz et al., 2006) and Methylobacterium (Torgonskaya et al., 2019). The aerobic pathway for DCM biodegradation begins with a glutathione S-transferase producing formaldehyde, which is partially oxidized to CO2 and chloride (Muller et al., 2011). To date, anaerobic biodegradation of DCM has been solely reported for bacteria affiliated with the Peptococcaceae family: Dehalobacterium, Dehalobacter, and Candidatus Dichloromethanomonas elyunquensis (D. elyunquensis). Dehalobacterium formicoaceticum (Dhb f.) is the only pure culture described using DCM as carbon and energy source under anoxic conditions, and produces formate and acetate as end products (Mägli et al., 1998). Recently, mixed bacterial cultures containing Dehalobacter sp. and/or D. elyunquensis have been reported to ferment DCM to mainly acetate (Chen et al., 2017; Justicia-Leon et al., 2012; Kleindienst et al., 2017; Lee et al., 2015, 2012).

Besides direct release into groundwater, DCM can also be produced by dehalogenation of the higher chlorinated methanes (CMs), i.e. carbon tetrachloride (CT) or chloroform (CF). The latter is listed as a probable carcinogen and a priority contaminant as well (ATSDR, 2016; European Commission, 2012; IARC, 2018). Under anoxic conditions, DCM might derive either from the hydrogenolysis of CF by the organohalide-respiring bacteria Dehalobacter sp. and Desulfitobacterium sp. (Chan et al., 2012; Ding et al., 2014; Tang and Edwards, 2013; Wong et al., 2016). Also, the anaerobic degradation of CF by Acetobacterium sp. comprises both a reductive branch leading to DCM and an oxidative pathway leading to CO2 (Egli et al., 1990, 1988; Wanner et al., 2018). DCM can be also produced co-metabolically by, for example, Methanosarcina sp. or Clostridium sp. (Egli et al., 1988; Gälli and McCarty, 1989; Krone et al., 1989b, 1989a; Mikesell and Boyd, 1990).

Chlorinated solvents are commonly found in complex mixtures in groundwater from contaminated sites rather than as individual chemicals. In multi-contaminated aquifers, microbial degradation can be affected and even inhibited by co-contaminants. This information is relevant to foresee the efficiency of natural attenuation or enhanced bioremediation strategies. For instance, the inhibitory effects of CF on key microbial processes such as methanogenesis and microbial dechlorination reactions of chlorinated ethenes (CEs) and ethanes are well known (Weathers and Parkin, 2000; Wei et al., 2016). In addition, it has also been reported that CT and CF inhibit their mutual biodegradation (da Lima and Sleep, 2010; Grostern et al., 2010; Justicia-Leon et al., 2014).

Compound-specific isotope analysis (CSIA) is a useful tool that allows bioremediation practitioners to assess the effectiveness of remediation treatments. During chemical or biological degradation of target contaminants, bonds containing the lighter isotopes are preferentially broken, causing the remaining contaminant to be enriched in the heavier isotopes compared to the original isotopic value  (Aelion et al., 2009). This is quantified through the abundance ratio of specific stable isotopes (e.g. 13C/12C, 37Cl/35Cl) in contaminant molecules relative to an international standard (Aelion et al., 2009). This technique may be used for source apportionment as well as the monitoring of transformation processes in the field (Elsner, 2010). In addition, a quantitative estimation of contaminant transformation extent in the field can be possible, provided that the isotope fractionation (ε) for a given compound and degradation pathway are known and well constrained (Aelion et al., 2009).

Dual element isotope analysis has some advantages over a single element isotope approach. On the one hand, by improving the identification of the source (either related to commercial solvents release or from parent compound degradation); on the other, by allowing the elucidation of the fate of pollutants in the field. During biodegradation processes, single element intrinsic isotope effects related to C–Cl bond cleavage can be masked due to the occurrence of previous rate-limiting steps such as preceding enzymatic reactions (Sherwood Lollar et al., 2010), bioavailability of electron donor and/or acceptor (Aeppli et al., 2009; Kampara et al., 2008; Thullner et al., 2008), substrate uptake and transport through the cell membrane (Cichocka et al., 2007; Renpenning et al., 2015), among others. These processes are generally non- or slightly-isotope fractionating so that both elements (e.g. C and Cl) are affected similarly. In this case, by taking the ratio of the isotope shift for the two elements (e.g. Λ = 13C / 37Cl), their masking effect is cancelled out and these slopes reflect better the ongoing degradation mechanisms (Elsner, 2010). While single element isotope fractionation could provide insight into the underlying reaction mechanisms in laboratory biodegradation experiments, this is not possible under field conditions. The reason is that contaminant concentration changes at the field are also related to processes other than its transformation (such as sorption and hydrodynamic dispersion), preventing accurate calculation of ε values. Thus, Λ values determined from laboratory studied reactions can be compared with those obtained from groundwater samples to investigate degradation processes at the field scale (Badin et al., 2014; Hermon et al., 2018; Hunkeler et al., 2009; Palau et al., 2017; Rodríguez-Fernández et al., 2018b).

Several laboratory studies have applied dual C–Cl isotope analysis to describe biotic and abiotic transformation mechanisms for DCM (Chen et al., 2018; Heraty et al., 1999; Torgonskaya et al., 2019) and CF (Heckel et al., 2019, 2017a; Rodríguez-Fernández et al., 2018a; Torrentó et al., 2017) but, to the best of the author’s knowledge, the application of a dual isotope approach in DCM-contaminated sites has not been reported yet.

The purpose of the work presented herein is two-fold. First, it aims to determine i) the ΛC/Cl value for the anaerobic degradation of DCM by a Dehalobacterium-containing culture, and ii) the carbon and chlorine isotopic compositions (δ13C, δ37Cl) of several commercial pure phase DCM and CF. These new isotope data enrich the database to which dual C–Cl isotopes slopes measured in the field can be compared to elucidate degradation mechanisms. Second, a multi-method approach was used at two industrial sites impacted by DCM, CF, trichloroethene (TCE) and mono-chlorobenzene (MCB) to investigate their origin, fate and intrinsic biodegradation potential. To this end, hydrochemical conditions in groundwater, concentrations of target contaminants and their stable isotope ratios (δ13C, δ37Cl) in field samples were analysed. Afterwards, a dual C–Cl isotopic assessment using the results from this study and reference values from the literature was performed to reveal the origin and fate of DCM. Lastly, field-derived microcosms testing for biostimulation as well as bioaugmentation, and 16S rRNA high-throughput sequencing were performed to assess the intrinsic biodegradation potential of target contaminants at the sites.

- Materials and methods

  - Materials

Pure phase chlorinated compounds isotopically characterised and/or used for the experiments were purchased at the highest purity available from the following suppliers: CF (Sigma Aldrich, Alfa, Merck), DCM (Sigma Aldrich, Fisher, SDS, Merck), MCB (Sigma Aldrich) and TCE (Panreac). CF (Sigma Aldrich) was used for the enrichment of field-derived cultures. Industrial grade sodium L-lactate (97% purity, at 60% w/w) was from Purac (Corbion). All other chemicals and reagents used were of the highest purity available.

Samples of the time-course degradation of DCM in a culture containing Dehalobacterium (Dhb) were obtained from a previous study reported elsewhere (Trueba-Santiso et al., 2017).

Two different commercial bacterial consortia were used for the microcosm bioaugmentation tests. Detailed information about the products cannot be disclosed due to commercial confidentiality reasons. Commercial inoculum “S” was reported to degrade CF to DCM, and fully dechlorinated CEs to ethene (ETH). Commercial inoculum “M” was reported to degrade CF to methane via DCM and chloromethane.

  - Hydrogeochemistry of the studied sites

The sites are located in the Catalan Coastal Ranges, which are characterized by an echelon fault system, subparallel to the coast, that affected the Hercynian basement and the Mesozoic overlying sediments. The Neogene extension created a "horst and graben" system filled by Miocene and Quaternary sediments. Both sites are located in the Neogene basin, one of them in the Miocene sediments and the other in the granitic basement.

Site 1

The studied site is located at an industrial area in the Barcelona province (Spain). Geologically, it is constituted by clays and conglomerates of quaternary age overlying unconformable upper Miocene siliciclastic sediments that dip 15–20º in the NW direction. Petrologically, the Miocene sediments are represented by clay and sands composed of abundant quartz grains, mica and kaolinized feldspars and, locally, by some conglomerates. Hydrogeologically, Miocene aquifer has an average transmissivity of 5–180 m2/d, less than the alluvial aquifer. A geological cross section of the studied area can be found in Section A of the Supporting Information (SI).

A previous characterization of the site detected high loads of contaminants and dense nonaqueous phase liquid (DNAPL) in groundwater, which was originated after improper storage of the substances (Figure 1A, SI section A). The main organic halogenated compounds included DCM, CF, TCE, and MCB, but cis-1,2-dichloroethene (cis-DCE), vinyl chloride (VC), tetrachloroethene (PCE), acetone, BTEX, and tetrahydrofuran were also detected. The contamination plume was considered finite and contained. After the initial site characterization, a pump and treat (P&T) remediation system (groundwater extraction) was implemented. Under this ongoing treatment, groundwater flowed radially towards the extraction points and a total of seven conventional fully screened monitoring wells were sampled (Figure 1A, SI section B).

Site 2

This site is also located at an industrial zone in the Barcelona province (Spain), but more than 30 km apart from Site 1. Geologically, it is constituted by Quaternary alluvial sediments represented by sands and gravels that overlie unconformably the basement of the Neogen basin made by later-Hercynian intrusive granites. Hydrogeologically, groundwater flows mainly from the alluvial gravels to the river freshwater and, to a lesser extent, through the more fractured and weathered granite. A conceptual site model of the studied area can be found in SI section C.

Like in Site 1, it was previously confirmed that the aquifer in Site 2, originally contaminated because of improper management, contained DNAPL (Figure 2A, SI section C). The most abundant contaminants in groundwater included DCM, CF, TCE, and MCB, but acetone, toluene, PCE, CT, VC, tetrahydrofuran and benzene were also detected in groundwater. After the initial site characterization, dual-phase extraction (DPE) and P&T remediation systems were implemented and ongoing during this study. For the hydrochemical characterization, nine conventional fully screened monitoring wells were sampled (Figure 2A, SI section D). At that time, the groundwater flow direction was E–W, and radially towards the extraction points due to DPE and P&T, and the contamination source was reported to be still active (SI section D).

  - Collection of groundwater samples

Hydrochemical parameters (Eh, pH, T and electric conductivity) were measured in-situ and groundwater samples from selected monitoring wells (see SI sections B and D) were collected as described elsewhere (Blázquez-Pallí et al., 2019). It has to be taken into account that these values are an average of the screen length. Samples for chemical and isotopic analysis were collected in different sampling containers on July 11th and 12th, 2017, from Site 1 and Site 2. Groundwater was collected with a peristaltic pump or bailer, depending on the water table depth. Samples for C–Cl CSIA were preserved with HNO3 (pH~2) (Badin et al., 2016) to prevent biodegradation processes. For the microcosm experiments, groundwater with fine sediments from PZ-9, PZ-19 and PZ-36 from Site 2 (SI section D) and PI-2 from Site 1 (SI section B) was sampled on the 18th and 19th of June 2018, respectively. All samples were collected in amber glass bottles sealed with PTFE caps and stored in the dark at 4ºC until used.

  - Set up of laboratory microcosms

The microcosms were prepared within the following two days after sampling. They consisted of 65 mL of sampled groundwater in 100 mL glass serum sterile bottles sealed with Teflon-coated butyl rubber septa as described elsewhere (Blázquez-Pallí et al., 2019). To investigate whether a potential bioremediation treatment would be feasible at the investigated sites, three different experiments were prepared in triplicate: (i) a control containing only groundwater from the site, which tested for monitored natural attenuation (MNA); (ii) groundwater with sodium lactate (~4 mM), which tested for biostimulation; and (iii) groundwater inoculated with the commercial bacterial consortia described above (106–107 cells/mL) plus sodium lactate (~4 mM), which tested for bioaugmentation. For the samples from wells PZ-19 and PZ-36 (Site 2, SI section D), two different bioaugmentation tests were established, i.e. one with each commercial inoculum, whereas only one bioaugmentation test (using the commercial inoculum “S”) was performed with samples from PZ-9 (Site 2, SI section D) and PI-2 (Site 1, SI section B).

To identify the bacteria responsible for CF degradation, the dilution-to-extinction method (Löffler et al., 2005) was applied in 12-mL vials containing an anoxic defined medium described elsewhere (Martín-González et al., 2015) and using CF as electron acceptor. After four extinction series, the more diluted vial showing activity against CF was used as inoculum for serum bottle microcosms, which were selected for 16S rRNA analysis after consuming 1 mM CF. The serum bottle microcosms were prepared as described elsewhere (Martín-González et al., 2015).

  - DNA extraction and 16S rRNA gene amplicon sequencing

Biomass was harvested from whole serum bottle microcosms in sterile falcon tubes that were centrifuged for 40 min at 7000 × g and 10 ºC in an Avanti J-20 centrifuge. The pellets were resuspended in sterile PBS buffer and, afterwards, DNA was extracted with the Gentra Puregene Yeast/Bact kit (Qiagen) following the instructions of the manufacturer. Extraction and analysis of the DNA of each microcosm was performed separately. The regions V3–V4 of the 16S rRNA genes were amplified with primers S-D-Bact-0341-b-S-17 and S-D-Bact-0785-a-A-21 (Klindworth et al., 2013) with the Illumina MiSeq platform at Serveis de Genòmica i Bioinformàtica from Universitat Autònoma de Barcelona.

  - Analytical methods

The hydrogeochemical parameters of groundwater (temperature, pH, redox potential (Eh) and electric conductivity) were determined in-situ using a multiparameter probe 3430 WTW (Weilheim) as described elsewhere (Blázquez-Pallí et al., 2019).

Chlorinated compounds were quantified by analysing 500 µL of headspace samples by gas chromatography (GC) coupled to a flame ionization detector (FID), as described elsewhere (Martín-González et al., 2015). Lactate, acetate and other short-chain fatty acids (VFAs) were monitored by HPLC from 1 mL filtered liquid samples (0.22 μm, Millex), as previously reported (Mortan et al., 2017).

Stable carbon isotope ratios (13C) of pure in-house standards were determined with a Flash EA1112 (Carlo-Erba, Milano, Italy) elemental analyser (EA) coupled to a Delta C isotope ratio mass spectrometer (IRMS) through a Conflo III interface (Thermo Finnigan, Bremen, Germany), while the 13C values of target chlorinated compounds (DCM, CF, CEs and MCB) in field samples were obtained by CSIA using an Agilent 6890 GC coupled to a Delta Plus IRMS, as described in Blázquez-Pallí et al. (2019). All analyses (standards and experimental samples) had a one standard deviation (1σ) lower than 0.5‰.

Stable chlorine isotope ratios (37Cl) were obtained by CSIA at Isotope Tracer Technologies Inc., Waterloo (ON, Canada). A 6890 GC (Agilent, Santa Clara, CA, U.S.) coupled to a MAT 253 IRMS (Thermo Finnigan, Bremen, Germany) were used. This IRMS, equipped with nine collectors, is a continuous flow IRMS with a dual-inlet (DI) mode option. The DI bellows are used as the monitoring gas reservoir (either CF or DCM) and reference peaks were introduced at the beginning of each analysis run (Shouakar-Stash et al., 2006). For the analysis of chlorine isotope ratios of CF, the two main ion peaks (m/z 83 and 85) were used, which correspond to isotopologue pairs that differ by one heavy chlorine isotope ([35Cl212C1H]+ and [37Cl35Cl12C1H]+, respectively) (Breider and Hunkeler, 2014). For DCM, the chlorine isotopic composition was determined from two ion peaks of the molecular group (m/z 84 and 86) corresponding to [35Cl212C1H2]+ and [37Cl35Cl12C1H2]+, respectively. Similarly to carbon isotopes analysis, the analytes were extracted by headspace solid-phase microextraction, as shown elsewhere (Palau et al., 2017). Samples and standards were diluted at similar concentrations and measured in duplicate. The δ37Cl values of pure in-house standards were characterized relative to SMOC (Standard Mean Ocean Chloride) by offline IRMS analysis after conversion of DCM to methyl chloride according to Holt et al. (1997). These standards were later used to correct all measurements from samples. Precision (1σ) of the analysis was ≤ 0.2‰ for 37Cl.

The isotopic compositions of carbon and chlorine are reported in delta notation (δhE, in ‰), relative to the international standards VPDB (Vienna Pee Dee Belemnite) and SMOC (Coplen, 1996; Kaufmann et al., 1984), respectively (Eq. 1),

δhE=RsampleRstd-1       (1)

where Rsample and Rstd are the isotope ratios (i.e. 13C/12C, 37Cl/35Cl) of the sample and the standard of an element E, respectively (Elsner, 2010).

  - Calculations for interpretation of isotopic data

A simplified version of the Rayleigh equation in the logarithmic form (Eq. 2) correlates changes in the isotopic composition of an element in a compound (Rt/R0) with changes in its concentration for a given reaction (f = Ct/C0), which allows to determine the corresponding isotopic fractionation (ε) (Coplen, 2011; Elsner, 2010).

lnRtR0=ε·lnf       (2)

Rt/R0 can be expressed as (δhEt + 1) / (δhE0 + 1) according to δhE definition. For the experiment of DCM degradation by the stable enrichment culture containing Dhb, the εCl was obtained from the slope of the linear regression according to Eq. 2. The uncertainty corresponds to the 95% confidence interval (CI).

For a target contaminant, the extent of degradation (D%) in the field was evaluated with Eq. 3, which is derived from the Rayleigh equation (Eq. 2),

D%=1-δhEt+1000δhE0+10001000εE·100       (3)

where εE refers to the isotopic fractionation, δhEt are the isotope data from groundwater samples and δhE0 is the most depleted value found at the field site, assumed to be the most similar to the original source. To this regard, differences in isotope values in the field for both carbon and chlorine (Δδ13C, Δδ37Cl) must be >2‰ for degradation to be considered significant (Hunkeler et al., 2008). Laboratory derived ε values can either be site-specific (i.e., from microcosm experiments prepared with soil and/or groundwater from the contaminated site) or obtained from the literature (Table 1).

Finally, the ΛC/Cl value for the DCM biodegradation experiment was obtained from the slope of the linear regression in the dual C–Cl isotope plot (Elsner, 2010) and the uncertainty is the 95% CI.

  - Dual C–Cl isotopic assessment based on selected ΛC/Cl

To understand the origin and fate of DCM and CF at each site, a dual C–Cl isotopic assessment (Badin et al., 2016; Hunkeler et al., 2008) was performed based on the measured δ13C and δ37Cl data of field and commercial compounds, the definition of D% (Eq. 3), and εC, εCl and ΛC/Cl from the available literature (Tables 1 and 2). The procedure followed for the dual C–Cl isotopic assessment is detailed in SI section E.

- Results and discussion

  - δ13C and δ37Cl of pure in-house commercial standards

Several pure in-house commercial standards were analysed to determine their δ13C and δ37Cl compositions (see details in Table 2). δ13CDCM for the standards that were measured in this study ranged from -35.4 ± 0.2 to -41.71 ± 0.08‰, while δ37ClDCM ranged from +2.1 ± 0.3 to -2.7 ± 0.1‰. δ13CCF ranged from -47.96 ± 0.06 to -52.7 ± 0.1‰, while δ37ClCF ranged from -2.6 ± 0.1 to -5.9 ± 0.2‰. A significant variation was observed among the different DCM and CF commercial standards analysed in this study, but also when compared to some of the values available in the literature. For instance, the δ37ClDCM value obtained for the Fisher standard (-2.7 ± 0.1‰) is more depleted in 37Cl than the values available to date (Table 2), emphasizing the relevance of reference isotopic composition of the pure products provided in this study for future data interpretation. Lastly, δ13C for TCE and MCB was also analysed here and resulted in -25.40 ± 0.03‰ and -27.09 ± 0.07‰, respectively.

  - Chlorine isotope fractionation and ΛC/Cl during anaerobic DCM degradation by a Dhb-containing culture

Samples of a Dhb-containing culture that were previously killed to analyse the decrease of concentration and the carbon isotope fractionation of DCM (Trueba-Santiso et al., 2017) were used in this study. The δ37Cl during DCM degradation increased from -5.7 ± 0.2 to +7.2 ± 0.2‰ after 97% transformation, but neither changes in concentration nor in δ37Cl (average -5.7 ± 0.2‰) were observed in the abiotic controls. However, this last point (97%) was excluded because it appeared to be deviated from the linear regression (R2=0.95, Eq. 2), whereas the best-fitting was obtained for δ37Cl values up to 84% degradation (R2=0.98) leading to a εCl of -5.2 ± 0.6‰ (Figure 3A). This phenomenon was observed previously by Mundle et al. (2013), who reported higher uncertainty in ε values calculated at later stages of the reaction and suggested examining the linearity of the fits over shorter reaction progress intervals. To date, there is still no εCl value available for Dehalobacter, but the εCl value of -5.2 ± 0.6‰ obtained in this study for Dhb is not significantly different from those reported for D. elyunquensis (-5.2 ± 0.1‰), Dhb f. (-5.3 ± 0.1‰), and purified dehalogenases of M. extorquens DM4 and Methylophilus leisingeri DM11(-5.7 and -5.6‰, respectively) (Chen et al., 2018; Torgonskaya et al., 2019) according to the student’s t-test (8 degrees of freedom, p=0.05). In contrast, it is significantly different than those reported for Hyphomicrobium sp. strain MC8b (-3.8‰) and Methylobacterium extorquens DM4 (-7.0‰) (Heraty et al., 1999; Torgonskaya et al., 2019) (Table 1).

The εC recalculated up to 84% degradation for Dhb (-31 ± 3‰, R2=0.986) does not vary much from the previously published value (-27 ± 2‰, R2=0.985) (Trueba-Santiso et al., 2017), and still differ from the reported DCM-fermentative bacteria Dehalobacter (-16 ± 2‰) (Lee et al., 2015). However, recent studies on Hyphomicrobium sp. strains (Hermon et al., 2018) and Methylobacterium extorquens DM4 (Torgonskaya et al., 2019) has widened the εC range attributed to methylotrophic bacteria (-22 to -66.6‰), not allowing to distinguish hydrolytic transformation of DCM via glutathione-dependent dehalogenases and fermentation pathway just by carbon isotopes.

A very good linear correlation (R2=0.995) was also obtained for δ13C against δ37Cl values up to 84% degradation in a dual C–Cl isotope plot (ΛC/Cl of 5.9 ± 0.3, Figure 3B). This ΛC/Cl value by the Dhb-containing culture is significantly different from those reported for D. elyunquensis (ΛC/Cl = 3.40 ± 0.03) and for Dhb f. (ΛC/Cl = 7.89 ± 0.12) (Chen et al., 2018) (Figure 3B) according to the student’s t test (8 degrees of freedom, p=0.05). The difference between Dhb and D. elyunquensis is in accordance with recent proteogenomic findings suggesting that both genera have distinct DCM degradation pathways (Kleindienst et al., 2019). However, the difference observed between Dhb and Dhb f. may suggest that they either have mechanistic disparities during DCM degradation, even though they both belong to the same genus. Higher ΛC/Cl values, ranging from 8.1 to 11.2, have been reported for the aerobic Hyphomicrobium sp. strain MC8b (Heraty et al., 1999), Methylobacterium extorquens DM4, and DM4 and DM11 DCM dehalogenases (Torgonskaya et al., 2019). To date, these values hint at the distinction between aerobic (11.2 to 8.1) and anaerobic (7.89 to 3.40) pathways (Figure 3B). However, further research on this topic would be needed to confirm such hypothesis. See Table 1 for a detailed review of literature values.

  - Field sites investigation

    - Site 1

The studied aquifer exhibited mixed redox conditions (Eh ranged from +250 to -52) mV), but most of the wells were in anoxic conditions (SI section F), which are conducive to reductive dechlorination reactions. Temperature and pH were considered homogeneous throughout the aquifer (21.1 ± 0.7 ºC and 6.8 ± 0.3, respectively) (SI section F). CF was the organohalide detected at the highest concentration (454 mg/L), followed by MCB (82.7 mg/L), TCE (47.2 mg/L), and DCM (1.47 mg/L) (Figure 1A and SI section G).

The C–Cl isotopic composition of CF could be determined in 5 out of 7 monitoring wells (Figure 1A). Obtained δ13C values for CF showed a relatively small but significant variation (Δδ13CCF) of 3.2‰ between wells PI-17 and CV-5, which exhibited the most enriched and depleted δ13CCF, respectively (SI section H). Since the difference in the carbon isotope composition was ˃2‰, it could be related to CF transformation in groundwater (Hunkeler et al., 2008). In contrast, the maximum Δδ37ClCF observed at the site between wells CV-5 and PI-24 was lower than 2‰. This agrees well with previous studies showing εC values usually greater than εCl during CF biodegradation (Table 1). The dual isotopic composition of CF in the analysed groundwater samples fell outside the isotopic range of commercial CF, except for samples collected in well CV-5 (SI section I). This result would further support a potential transformation of CF at the site.

For DCM, only one C–Cl isotopic composition could be determined (in well PI-26) due to DCM concentration being too low for carbon CSIA, but δ37Cl could be determined in 4 out of 7 wells (Figure 1A). Obtained δ37Cl for DCM showed a significant variation in 37Cl/35Cl ratios of Δδ37ClDCM = 2.3‰ between wells CV-5 and PI-26 (SI section H, Figure 1A), and these δ37Cl were enriched compared to those of commercial DCM (Table 2). As mentioned earlier, no Δδ13CDCM can be provided for δ13C since only one value could be obtained, but this value was also enriched compared to most of the δ13C reported for commercial DCM (Table 2). Hence, this dual C–Cl isotopic composition for DCM in the analysed groundwater of well PI-26 fell outside the isotopic range of commercial DCM, suggesting a potential transformation of DCM at the site as well (SI section I). When comparing these results to the literature, the low carbon enrichment observed could indicate that the bacteria responsible for DCM degradation have a εC that is in the lower range (e.g. Dehalobacter sp., which has the lowest reported εC as presented in Table 1). An additional evidence pointing towards in-situ DCM biodegradation was the detection of acetate in wells PI-26 and CV-3 (Figure 1A and SI section G), which is the main by-product of DCM fermentation (Justicia-Leon et al., 2012; Lee et al., 2015; Mägli et al., 1998; Trueba-Santiso et al., 2017).

Interestingly, δ13CTCE in the wells that also contained CF ranged between -32.5 and -31.8‰ but was significantly enriched in 13C in the unique well (PI-3) that lacked CF (δ13CTCE = -28.2‰, Figure 1A). This may be in agreement with several studies suggesting that CF can inhibit significantly microbial reductive dechlorination of CEs, as reviewed elsewhere (Wei et al., 2016). However, cis-DCE, VC and ETH were also detected in most of the monitored wells (SI section G) indicating that, despite this potential inhibition effect suggested by the isotopic analysis, full dechlorination of TCE occurred.

Lastly, no significant carbon isotope fractionation was observed for MCB (Δδ13CMCB < 2‰, SI section H), and obtained δ13C values were within the available range of commercial MCB solvents (Table 2), indicating that this compound was, most likely, not being degraded.

To investigate the biodegradation potential of the site, three different microcosm treatments were prepared with groundwater from well PI-2 (SI section J). The non-amended microcosms used as natural attenuation controls fully degraded CF and DCM, and transformed TCE to VC after 75 days. The microcosms amended with either only lactate (biostimulation) or also inoculated with “S” (bioaugmentation) increased the degradation rate of CF (to DCM), DCM, TCE and, more importantly, fully dechlorinated the latter to stoichiometric amounts of ETH. Both CMs and CEs elimination occurred simultaneously (SI section J). These results confirmed the feasibility of in-situ CF, DCM and TCE biodegradation, and agreed with the information obtained from the isotopic field data that was just discussed. In contrast, MCB always accumulated in the medium of the microcosms and was not degraded (SI section J). Such evidence further supports the evidence obtained from the isotopic field data, which pointed towards the recalcitrance of the pollutant. Thus, it could be discarded that benzene, which was detected in the preliminary characterization of this aquifer, was derived from MCB biodegradation. Regarding the amended substrate, lactate was converted to acetate (major product) and propionate in all the amended microcosms (SI section K).

The composition of the microbial community was assessed by 16S rRNA high-throughput sequencing. The analysis was performed on groundwater samples collected from well PI-2 that were enriched with CF for four consecutive dilution series. The taxonomic assignments of 16S rRNA sequences revealed that the four predominant genera were Clostridium (31%), Sedimentibacter (25%), Dehalobacter (10%) and Acetobacterium (4%) (SI section L). The presence of Dehalobacter, a well-known CF and DCM degrader (Justicia-Leon et al., 2012; Lee et al., 2015), further supports the intrinsic biodegradation potential of the aquifer to remediate CMs. In addition, results also suggested that Clostridium sp. and Acetobacterium sp. could be somehow involved in the degradation of CF as well, as described elsewhere (Egli et al., 1990, 1988; Gälli and McCarty, 1989; Wanner et al., 2018).

With these evidences and aiming to better understand the origin and fate of CF and DCM at this site, a dual C–Cl isotopic assessment was performed based on the assumption that CF in well CV-5 reflected the initial isotopic signature (CF0) since it possessed the most depleted δ13C and δ37Cl values (Figure 1A). The dual isotope fractionation pattern observed for CF in CV-3, PI-17 and PI-26 was consistent with the degradation of leaked commercial CF, since the data points fell within the estimated potential isotopic composition for degraded CF (orange area, Figure 1B). Accordingly, the values for DCM initially produced from CF degradation (DCMini) ranged between -50.9‰ and -73.9‰ for δ13C and was of -2.2‰ for δ37Cl (Figure 1B, SI section E). The isotopic composition of DCM detected in well PI-26 (SI section H) was enriched in 37Cl compared to that of commercial DCM (Table 2), and in both 13C and 37Cl compared to that of DCM produced by CF degradation (SI section E), suggesting that it could be subject to biodegradation (Figure 1B). However, regarding the main source of detected DCM at the site, obtained results point to a potential spill of DCM at the site being more likely than DCM produced by CF degradation, but neither could be confirmed.

In the above scenario, and assuming Dehalobacter sp. was the responsible for CF biotransformation, the extent of biodegradation (D%) for CF estimated in the wells CV-3, PI-17 and PI-26 (where Δδ13C was > 2‰, SI section H) ranged between 7–11% considering the εC data reported for Dehalobacter strain CF (εC = -28 ± 2)). Data for Dehalobacter strain CF was used for this calculation because its ΛC/Cl was more coherent with the field results from Site 1 (i.e. the isotopic pattern of positive dual C–Cl slopes observed for CF measurements), than the ΛC/Cl of Dehalobacter strain UNSWDHB (Table 1). Nevertheless, further research needs to be done on this culture to better understand the roles of Dehalobacter, Clostridium and Acetobacterium bacteria, and elucidate the biodegradation mechanisms of both CF and DCM, which would allow an improved assessment and quantification of D% at the field.

    - Site 2

At this site, Eh ranged from +254 to -118 mV, but groundwater in most of the wells exhibited oxic conditions which limit the occurrence of reductive dechlorination reactions (SI section F). Temperature and pH were considered homogeneous and averaged 24 ± 2 ºC and 6.8 ± 0.3, respectively (SI section F). CF was detected at the highest concentrations (272 mg/L), followed by DCM (27 mg/L), TCE (1.7 mg/L) and MCB (0.88 mg/L) (Figure 2A and SI section G).

δ13C and δ37Cl of CF could be determined in 7 out of 9 wells because the concentration of CF in PZ-5 and PZ-31 was below the limit of quantification (Figure 2A). The Δδ13CCF > 2‰ observed between PZ-9 and PZ-18 wells and the Δδ37ClCF < 2‰ observed between PZ-10 and PZ-18 wells (SI section H) could be related to CF transformation in groundwater (Hunkeler et al., 2008) considering the low εCl reported for CF transformation.  However, most of the C–Cl isotope compositions of CF were within the isotopic range for commercial CF (SI section I), which could also indicate that measured δ13C and δ37Cl belonged to a leaked and not degraded CF.

On its side, detected DCM presented a higher variation for both C and Cl isotope ratios (Δδ13CDCM = 6.1‰ between wells PZ-19 and PZ-36, Δδ37ClDCM = 2.2‰ between wells PZ-9 and PZ-18, Figure 2A, SI section H) indicative of potential in-situ transformations. In this case, however, the isotopic composition in well PZ-36 did fall within the commercial DCM, whereas well PZ-19 did not (Figure 2B and SI section I, see further discussion below).

For TCE, Δδ13C was of 5.8‰ (SI section H), which could be related to TCE transformation in groundwater (Hunkeler et al., 2008), and measured δ13CTCE values were more enriched in 13C than the δ13C compositions of commercial TCE (Table 2), in agreement with potential TCE degradation. However, the reductive dechlorination products of TCE (DCE, VC, and ETH) were detected occasionally and at very low concentrations (SI section G).

Microcosm experiments were prepared with groundwater from wells PZ-9, PZ-19 and PZ-36. Microcosms for well PZ-9 did not show significant degradation of neither CF nor CEs, and no major differences were observed between the non-amended, biostimulated and the bioaugmented treatments after 150 days (SI section M). However, traces of DCM and VC, a slight decrease in PCE and TCE, and formation of trans-DCE were observed throughout the course of the study (SI section M). For PZ-19, no significant elimination of contaminants was observed, although TCE did exhibit a decrease in all microcosms and traces of cis-DCE, trans-DCE and VC were detected in some of them (SI section N). For PZ-36, the non-amended, biostimulated and bioaugmented (with “M”) microcosms did neither degrade CF nor CEs significantly. In contrast, the bioaugmented (with “S”) treatment exhibited a CF decrease with the transient production of DCM, and TCE was completely transformed to VC and trans-DCE (SI section O). Amended lactate was mainly transformed to acetate in PZ-9 and PZ-36 microcosms. In PZ-19, however, lactate was not completely consumed but, in both bioaugmentation tests, its decrease was followed by acetate and propionate production (SI section K). The inability of inoculated bacteria to efficiently degrade contaminants in these microcosms suggested that there were inhibition issues, possibly due to the presence of co-contaminants in groundwater such as CT, which is a known inhibitor of the microbial activity and CF degradation in particular (section 2.2) (da Lima and Sleep, 2010; Grostern et al., 2010; Justicia-Leon et al., 2014; Wei et al., 2016). The results obtained with the microcosms suggested that, under the studied conditions, degradation of contaminants was very slow and inefficient, but possible to some extent, which agreed with the information obtained from the isotopic field data that was discussed above.

At this site, the dual C–Cl isotopic assessment was performed based on a CF0 signature of δ13C = -45.1‰ (from PZ-9) and δ37Cl = -3.5‰ (from PZ-10), as they were the most depleted values measured (Figure 2A). The dual isotope fractionation pattern observed for CF was not consistent with degradation (orange area, Figure 2B), in agreement with the microcosms. However, it should be noted that δ37ClCF values were enriched respect to CF0 signature. This could be attributable to Cl isotope fractionation processes in the unsaturated zone due to diffusion-controlled vaporization (Jeannottat and Hunkeler, 2012; Palau et al., 2016) or to the leakage of CF from different providers with distinct isotopic signatures. Moreover, it has to be taken into account that this release of CF was still active at the site and potential CF degradation could be masked by a more depleted CF input. In the hypothetical case that CF was biologically degraded, the calculated isotopic range for DCMini (formation of DCM from CF degradation) would range from -52.1‰ to -75.1‰ for carbon, and -3.5‰ for chlorine (Figure 2B, SI section E). The isotopic composition of DCM detected in groundwater (Figure 2B, SI section H) was very enriched in both 37Cl and 13C compared to that of initial DCM produced by CF degradation (grey area, Figure 2B). In detail, DCM detected in PZ-36 (with the highest DCM concentration, Figure 2A) was within the range of commercial DCM (Figure 2B), pointing to a potential release of DCM at the site. However, this DCM could be subject to a little biodegradation as acetate, the main by-product of DCM fermentation, was detected in the well PZ-36 and also in PZ-18 (SI section G). For the well PZ-19, the δ13CDCM value is slightly depleted in 13C while the δ37ClDCM value is lightly enriched in 37Cl compared to those measured in PZ-36 (Figure 2B). This could be explained similar to CF, in this case DCM could be an impurity in the CF raw source with isotopic signature changing over time due to change in CF providers or manufacturing processes or by the effect of vaporization and diffusion processes in the unsaturated zone (Jeannottat and Hunkeler, 2012; Palau et al., 2016). As noted above, the release of CF (and probably DCM) was still active at the site. Taking all these evidences into account, the extent of biodegradation (D%) for CF was not estimated since the results would not be representative of CF degradation.

- Conclusions

The present study provides additional data on isotope C and Cl compositions for different commercial CMs, widening the known range for these solvents and proving the utility of these results for data interpretation. In addition, the C and Cl isotopic fractionation values for the anaerobic degradation of DCM by a Dhb-containing culture were also determined. These results enrich the available database that can be used by practitioners to provide diagnostic information about CMs biodegradation in contaminated aquifers. The value of ΛC/Cl obtained for the investigated Dhb-containing culture was right in between of those described for Dhb f. and D. elyunquensis, which would allow the distinction of these DCM degradation mechanisms through isotope analyses.

The pumping regimes that were active at both studied sites probably impacted the geographical distribution of the chlorinated compounds and their corresponding isotopic signatures. For this reason, the focus of this work was not on the flow path of the plume but on the correlation between the isotopic composition of the chlorinated solvents and the biodegradation information obtained from microcosms experiments for each monitoring well. For this, the use of an integrative approach that combines C–Cl CSIA, laboratory microcosms, and 16S rRNA high-throughput sequencing demonstrated the intrinsic biodegradation potential of Site 1 to fully transform CF, DCM, and CEs, but not MCB. In contrast, results for Site 2 suggested that inhibition was preventing the efficient elimination of CMs and CEs at tested conditions. Nevertheless, the dual C–Cl isotopic assessment proved useful, as well, to elucidate the origin and fate of DCM and CF at both sites. Considering these results, a biostimulation (e.g. enhanced reductive dechlorination with lactate) could be applied at Site 1 to degrade both CMs and CEs. However, the persistence of MCB at this site, and the inhibition of biodegradation observed at Site 2, suggest that a treatment train (i.e. a combination or sequence of different remedial strategies targeting different groups of contaminants) could possibly be the optimal approach for the detoxification of these aquifers.

This study shows that such a multi-method approach allows for the collection of data that can help making decisions in the field. In this case, it is a valuable tool to evaluate the feasibility of biodegradation strategies to remediate chlorinated solvents in complex multi-contaminated aquifers, as it can provide the lines of evidence required to demonstrate whether bacteria can successfully detoxify groundwater, and identify any potential setbacks. The microcosm tests can provide a positive indication that complete dechlorination can be achieved by native microbial populations, and is useful to predict the effect of bioremediation treatments (biostimulation or bioaugmentation) to detoxify the aquifer. Changes in carbon and chlorine isotope composition among the residual fraction of the chlorinated compounds in the monitoring wells can provide field evidence for microbial degradation. Lastly, the positive molecular identification of key organohalide-respiring bacteria (e.g. Dehalobacter) can provide additional evidence that chlorinated solvents can be fully dechlorinated in the aquifer. Notwithstanding, there is a need for additional laboratory studies that correlate the metabolism of microbial transformations with stable isotope fractionation of multiple elements to support the interpretation of CSIA data obtained from contaminated groundwaters, as well as the potential inhibitory effect of co-contaminants over bacteria degrading organochlorides.

- Acknowledgements

This research has been supported by the Spanish State Research Agency (CTM2016-75587-C2-1-R and CGL2017-87216-C4-1-R projects), co-financed by the European Union through the European Regional Development Fund (ERDF). This work was also partly supported by Generalitat de Catalunya through the consolidate research groups (2017SGR-14 and 2017SGR-1733) and the Industrial Doctorate grant of N. Blázquez-Pallí (2015-DI-064). A. Trueba-Santiso acknowledges support from a MICINN predoctoral research grant (BES-2014-070817). M. Rosell acknowledges a Ramón y Cajal contract (RYC-2012-11920) from MICINN. The Departament d’Enginyeria Química, Biològica i Ambiental of Universitat Autònoma de Barcelona is a member of the Xarxa de Referència en Biotecnologia de la Generalitat de Catalunya. The authors thank Jesica M. Soder-Walz for her collaboration in the laboratory work.

References

Aelion, C.M., Höhener, P., Hunkeler, D., Aravena, R., 2009. Environmental Isotopes in Bioremediation and Biodegradation. CRC Press, Boca Raton. https://doi.org/10.1159/000076616

Aeppli, C., Berg, M., Cirpka, O.A., Holliger, C., Schwarzenbach, R.P., Hofstetter, T.B., 2009. Influence of mass-transfer limitations on carbon isotope fractionation during microbial dechlorination of trichloroethene. Environ. Sci. Technol. 43, 8813–8820. https://doi.org/10.1021/es901481b

ATSDR, 2016. Substance priority list [WWW Document]. Subst. Prior. List. URL https://www.atsdr.cdc.gov/SPL/index.html (accessed 10.31.17).

Badin, A., Broholm, M.M., Jacobsen, C.S., Palau, J., Dennis, P., Hunkeler, D., 2016. Identification of abiotic and biotic reductive dechlorination in a chlorinated ethene plume after thermal source remediation by means of isotopic and molecular biology tools. J. Contam. Hydrol. 192, 1–19. https://doi.org/10.1016/j.jconhyd.2016.05.003

Badin, A., Buttet, G., Maillard, J., Holliger, C., Hunkeler, D., 2014. Multiple dual C–Cl isotope patterns associated with reductive dechlorination of tetrachloroethene. Environ. Sci. Technol. 48, 9179–9186. https://doi.org/10.1021/es500822d

Blázquez-Pallí, N., Rosell, M., Varias, J., Bosch, M., Soler, A., Vicent, T., Marco-Urrea, E., 2019. Multi-method assessment of the intrinsic biodegradation potential of an aquifer contaminated with chlorinated ethenes at an industrial area in Barcelona (Spain). Environ. Pollut. 244, 165–173. https://doi.org/10.1016/j.envpol.2018.10.013

Breider, F., 2013. Investigating the origin of chloroform in soils and groundwater using carbon and chlorine stable isotopes analysis. Université de Neuchâtel (Switzerland).

Breider, F., Hunkeler, D., 2014. Investigating chloroperoxidase-catalyzed formation of chloroform from humic substances using stable chlorine isotope analysis. Environ. Sci. Technol. 48, 1592–1600. https://doi.org/10.1021/es403879e

Chan, C.C.H., Mundle, S.O.C., Eckert, T., Liang, X., Tang, S., Lacrampe-Couloume, G., Edwards, E., Sherwood Lollar, B., 2012. Large carbon isotope fractionation during biodegradation of chloroform by Dehalobacter cultures. Environ. Sci. Technol. 46, 10154–10160. https://doi.org/10.1021/es3010317

Chen, G., Kleindienst, S., Griffiths, D.R., Mack, E.E., Seger, E.S., Löffler, F.E., 2017. Mutualistic interaction between dichloromethane- and chloromethane-degrading bacteria in an anaerobic mixed culture. Environ. Microbiol. 19, 4784–4796. https://doi.org/10.1111/1462-2920.13945

Chen, G., Shouakar-Stash, O., Phillips, E., Justicia-Leon, S.D., Gilevska, T., Sherwood Lollar, B., Mack, E.E., Seger, E.S., Löffler, F.E., 2018. Dual carbon-chlorine isotope analysis indicates distinct anaerobic dichloromethane degradation pathways in two members of the Peptococcaceae. Environ. Sci. Technol. 52, 8607–8616. https://doi.org/10.1021/acs.est.8b01583

Cichocka, D., Siegert, M., Imfeld, G.G., Andert, J., Beck, K., Diekert, G., Richnow, H.H., Nijenhuis, I., 2007. Factors controlling the carbon isotope fractionation of tetra- and trichloroethene during reductive dechlorination by Sulfurospirillum ssp. and Desulfitobacterium sp. strain PCE-S. FEMS Microbiol. Ecol. 62, 98–107. https://doi.org/10.1111/j.1574-6941.2007.00367.x

Coplen, T.B., 2011. Guidelines and recommended terms for expression of stable-isotope-ratio and gas-ratio measurement results. Rapid Commun. Mass Spectrom. 25, 2538–2560. https://doi.org/10.1002/rcm.5129

Coplen, T.B., 1996. New guidelines for reporting stable hydrogen, carbon, and oxygen isotope-ratio data. Geochim. Cosmochim. Acta 60, 3359–3360. https://doi.org/10.1016/0016-7037(96)00263-3

da Lima, G.P., Sleep, B.E., 2010. The impact of carbon tetrachloride on an anaerobic methanol-degrading microbial community. Water. Air. Soil Pollut. 212, 357–368. https://doi.org/10.1007/s11270-010-0350-z

Ding, C., Zhao, S., He, J., 2014. A Desulfitobacterium sp. strain PR reductively dechlorinates both 1,1,1-trichloroethane and chloroform. Environ. Microbiol. 16, 3387–3397. https://doi.org/10.1111/1462-2920.12387

Egli, C., Stromeyer, S., Cook, A.M., Leisinger, T., 1990. Transformation of tetra- and trichloromethane to CO2 by anaerobic bacteria is a non-enzymic process. FEMS Microbiol. Lett. 68, 207–212. https://doi.org/10.1016/0378-1097(90)90152-G

Egli, C., Tschan, T., Scholtz, R., Cook, A.M., Leisinger, T., 1988. Transformation of tetrachloromethane to dichloromethane and carbon-dioxide by Acetobacterium-Woodii. Appl. Environ. Microbiol. 54, 2819–2824.

Elsner, M., 2010. Stable isotope fractionation to investigate natural transformation mechanisms of organic contaminants: principles, prospects and limitations. J. Environ. Monit. 12, 2005–2031. https://doi.org/10.1039/c0em00277a

European Commission, 2012. Priority Substances and Certain Other Pollutants according to Annex II of Directive 2008/105/EC [WWW Document]. URL http://ec.europa.eu/environment/water/water-framework/priority_substances.htm (accessed 10.31.17).

Gälli, R., McCarty, P.L., 1989. Biotransformation of 1,1,1-trichloroethane, trichloromethane, and tetrachloromethane by a Clostridium sp. Appl. Environ. Microbiol. 55, 837–844.

Gribble, G.W., 2010. Naturally Occurring Organohalogen Compounds - A Comprehensive Update, Asian Journal of WTO and International Health Law and Policy, Fortschritte der Chemie organischer Naturstoffe / Progress in the Chemistry of Organic Natural Products. Springer Vienna, Vienna. https://doi.org/10.1007/978-3-211-99323-1

Grostern, A., Duhamel, M., Dworatzek, S., Edwards, E., 2010. Chloroform respiration to dichloromethane by a Dehalobacter population. Environ. Microbiol. 12, 1053–1060. https://doi.org/10.1111/j.1462-2920.2009.02150.x

Heckel, B., Cretnik, S., Kliegman, S., Shouakar-Stash, O., McNeill, K., Elsner, M., 2017a. Reductive outer-sphere single electron transfer is an exception rather than the rule in natural and engineered chlorinated ethene dehalogenation. Environ. Sci. Technol. 51, 9663–9673. https://doi.org/10.1021/acs.est.7b01447

Heckel, B., Phillips, E., Edwards, E., Sherwood Lollar, B., Elsner, M., Manefield, M.J., Lee, M., 2019. Reductive dehalogenation of trichloromethane by two different Dehalobacter restrictus strains reveal opposing dual element isotope effects. Environ. Sci. Technol. 53, 2332–2343. https://doi.org/10.1021/acs.est.8b03717

Heckel, B., Rodríguez-Fernández, D., Torrentó, C., Meyer, A.H., Palau, J., Domènech, C., Rosell, M., Soler, A., Hunkeler, D., Elsner, M., 2017b. Compound-specific chlorine isotope analysis of tetrachloromethane and trichloromethane by gas chromatography-isotope ratio mass spectrometry vs gas chromatography-quadrupole mass spectrometry: method development and evaluation of precision and trueness. Anal. Chem. 89, 3411–3420. https://doi.org/10.1021/acs.analchem.6b04129

Heraty, L.J., Fuller, M.., Huang, L., Abrajano, T., Sturchio, N.., 1999. Isotopic fractionation of carbon and chlorine by microbial degradation of dichloromethane. Org. Geochem. 30, 793–799. https://doi.org/10.1016/S0146-6380(99)00062-5

Hermon, L., Denonfoux, J., Hellal, J., Joulian, C., Ferreira, S., Vuilleumier, S., Imfeld, G., 2018. Dichloromethane biodegradation in multi-contaminated groundwater: Insights from biomolecular and compound-specific isotope analyses. Water Res. 142, 217–226. https://doi.org/10.1016/j.watres.2018.05.057

Holt, B.D., Sturchio, N.C., Abrajano, T., Heraty, L.J., 1997. Conversion of chlorinated volatile organic compounds to carbon dioxide and methyl chloride for isotopic analysis of carbon and chlorine. Anal. Chem. 69, 2727–2733. https://doi.org/10.1021/ac961096b

Hunkeler, D., Meckenstock, R.U., Lollar, B.S., Schmidt, T.C., Wilson, J.T., 2008. A guide for assessing biodegradation and source identification of organic ground water contaminants using compound specific isotope analysis (CSIA), U.S. Environmental Protection Agency. Washington, D.C., EPA/600/R-08/148.

Hunkeler, D., Van Breukelen, B.M., Elsner, M., 2009. Modeling chlorine isotope trends during sequential transformation of chlorinated ethenes. Environ. Sci. Technol. 43, 6750–6756. https://doi.org/10.1021/es900579z

IARC, 2018. List of classifications, Volumes 1-122 [WWW Document]. IARC Monogr. Eval. Carcinog. Risks to Humans. URL https://monographs.iarc.fr/list-of-classifications-volumes/ (accessed 10.6.18).

IARC, 2016. Some chemicals used as solvents and in polymer manufacture, IARC Monographs on the Evaluation of Carcinogenic Risks to Humans. Lyon.

Jeannottat, S., Hunkeler, D., 2012. Chlorine and carbon isotopes fractionation during volatilization and diffusive transport of trichloroethene in the unsaturated zone. Environ. Sci. Technol. 46, 3169–3176. https://doi.org/10.1021/es203547p

Jendrzejewski, N., Eggenkamp, H.G.M., Coleman, M.. L., 2001. Characterisation of chlorinated hydrocarbons from chlorine and carbon isotopic compositions: Scope of application to environmental problems. Appl. Geochemistry 16, 1021–1031. https://doi.org/10.1016/S0883-2927(00)00083-4

Justicia-Leon, S.D., Higgins, S., Mack, E.E., Griffiths, D.R., Tang, S., Edwards, E., Löffler, F.E., 2014. Bioaugmentation with distinct Dehalobacter strains achieves chloroform detoxification in microcosms. Environ. Sci. Technol. 48, 1851–1858. https://doi.org/10.1021/es403582f

Justicia-Leon, S.D., Ritalahti, K.M., Mack, E.E., Löffler, F.E., 2012. Dichloromethane fermentation by a Dehalobacter sp. in an enrichment culture derived from pristine river sediment. Appl. Environ. Microbiol. 78, 1288–1291. https://doi.org/10.1128/AEM.07325-11

Kampara, M., Thullner, M., Richnow, H.H., Harms, H., Wick, L.Y., 2008. Impact of bioavailability restrictions on microbially induced stable isotope fractionation. 2. Experimental evidence. Environ. Sci. Technol. 42, 6552–6558. https://doi.org/10.1021/es702781x

Kaufmann, R., Long, A., Bentley, H., Davis, S., 1984. Natural chlorine isotope variations. Nature 309, 338–340. https://doi.org/10.1038/309338a0

Kleindienst, S., Chourey, K., Chen, G., Murdoch, R.W., Higgins, S.A., Iyer, R., Campagna, S.R., Mack, E.E., Seger, E.S., Hettich, R.L., Löffler, F.E., 2019. Proteogenomics reveals novel reductive dehalogenases and methyltransferases expressed during anaerobic dichloromethane metabolism. Appl. Environ. Microbiol. 85. https://doi.org/10.1128/AEM.02768-18

Kleindienst, S., Higgins, S.A., Tsementzi, D., Chen, G., Konstantinidis, K.T., Mack, E.E., Löffler, F.E., 2017. ‘Candidatus Dichloromethanomonas elyunquensis’ gen. nov., sp. nov., a dichloromethane-degrading anaerobe of the Peptococcaceae family. Syst. Appl. Microbiol. 40, 150–159. https://doi.org/10.1016/j.syapm.2016.12.001

Klindworth, A., Pruesse, E., Schweer, T., Peplies, J., Quast, C., Horn, M., Glöckner, F.O., 2013. Evaluation of general 16S ribosomal RNA gene PCR primers for classical and next-generation sequencing-based diversity studies. Nucleic Acids Res. 41, e1–e1. https://doi.org/10.1093/nar/gks808

Krone, U., Laufer, K., Thauer, R.K., Hogenkamp, H.P.C., 1989a. Coenzyme F430 as a possible catalyst for the reductive dehalogenation of chlorinated C1 hydrocarbons in methanogenic bacteria. Biochemistry 28, 10061–10065. https://doi.org/10.1021/bi00452a027

Krone, U., Thauer, R.K., Hogenkamp, H.P.C., 1989b. Reductive dehalogenation of chlorinated C1-hydrocarbons mediated by corrinoids. Biochemistry 28, 4908–4914. https://doi.org/10.1021/bi00437a057

Lee, M., Low, A., Zemb, O., Koenig, J., Michaelsen, A., Manefield, M.J., 2012. Complete chloroform dechlorination by organochlorine respiration and fermentation. Environ. Microbiol. 14, 883–894. https://doi.org/10.1111/j.1462-2920.2011.02656.x

Lee, M., Wells, E., Wong, Y.K., Koenig, J., Adrian, L., Richnow, H.H., Manefield, M.J., 2015. Relative contributions of dehalobacter and zerovalent iron in the degradation of chlorinated methanes. Environ. Sci. Technol. 49, 4481–4489. https://doi.org/10.1021/es5052364

Liang, X., Howlett, M.R., Nelson, J.L., Grant, G., Dworatzek, S., Lacrampe-Couloume, G., Zinder, S.H., Edwards, E., Sherwood Lollar, B., 2011. Pathway-dependent isotope fractionation during aerobic and anaerobic degradation of monochlorobenzene and 1,2,4-trichlorobenzene. Environ. Sci. Technol. 45, 8321–8327. https://doi.org/10.1021/es201224x

Löffler, F.E., Sanford, R.A., Ritalahti, K.M., 2005. Enrichment, cultivation, and detection of reductively dechlorinating bacteria. Methods Enzymol. 397, 77–111. https://doi.org/10.1016/S0076-6879(05)97005-5

Mägli, A., Messmer, M., Leisinger, T., 1998. Metabolism of dichloromethane by the strict anaerobe Dehalobacterium formicoaceticum. Appl. Environ. Microbiol. 64, 646–650.

Marshall, K.A., Pottenger, L.H., 2016. Chlorocarbons and Chlorohydrocarbons, in: Ley, C. (Ed) Kirk-Othmer Encyclopedia of Chemical Technology. John Wiley & Sons, Inc., Hoboken, NJ, USA, pp. 1–29. https://doi.org/10.1002/0471238961.1921182218050504.a01.pub3

Martín-González, L., Mortan, S.H., Rosell, M., Parladé, E., Martínez-Alonso, M., Gaju, N., Caminal, G., Adrian, L., Marco-Urrea, E., 2015. Stable carbon isotope fractionation during 1,2-dichloropropane-to-propene transformation by an enrichment culture containing Dehalogenimonas strains and a dcpA gene. Environ. Sci. Technol. 49, 8666–74. https://doi.org/10.1021/acs.est.5b00929

Mikesell, M.D., Boyd, S.A., 1990. Dechlorination of chloroform by Methanosarcina strains. Appl. Environ. Microbiol. 56, 1198–1201.

Mortan, S.H., Martín-González, L., Vicent, T., Caminal, G., Nijenhuis, I., Adrian, L., Marco-Urrea, E., 2017. Detoxification of 1,1,2-trichloroethane to ethene in a bioreactor co-culture of Dehalogenimonas and Dehalococcoides mccartyi strains. J. Hazard. Mater. 331, 218–225. https://doi.org/10.1016/j.jhazmat.2017.02.043

Muller, E.E.L., Bringel, F., Vuilleumier, S., 2011. Dichloromethane-degrading bacteria in the genomic age. Res. Microbiol. 162, 869–876. https://doi.org/10.1016/j.resmic.2011.01.008

Mundle, S.O.C., Vandersteen, A.A., Lacrampe-Couloume, G., Kluger, R., Sherwood Lollar, B., 2013. Pressure-monitored headspace analysis combined with compound-specific isotope analysis to measure isotope fractionation in gas-producing reactions. Rapid Commun. Mass Spectrom. 27, 1778–1784. https://doi.org/10.1002/rcm.6625

Nikolausz, M., Nijenhuis, I., Ziller, K., Richnow, H.H., Kastner, M., 2006. Stable carbon isotope fractionation during degradation of dichloromethane by methylotrophic bacteria. Environ. Microbiol. 8, 156–164. https://doi.org/10.1111/j.1462-2920.2005.00878.x

Palau, J., Jamin, P., Badin, A., Vanhecke, N., Haerens, B., Brouyère, S., Hunkeler, D., 2016. Use of dual carbon–chlorine isotope analysis to assess the degradation pathways of 1,1,1-trichloroethane in groundwater. Water Res. 92, 235–243. https://doi.org/10.1016/j.watres.2016.01.057

Palau, J., Yu, R., Mortan, S.H., Shouakar-Stash, O., Rosell, M., Freedman, D.L., Sbarbati, C., Fiorenza, S., Aravena, R., Marco-Urrea, E., Elsner, M., Soler, A., Hunkeler, D., 2017. Distinct dual C–Cl isotope fractionation patterns during anaerobic biodegradation of 1,2-dichloroethane: potential to characterize microbial degradation in the field. Environ. Sci. Technol. 51, 2685–2694. https://doi.org/10.1021/acs.est.6b04998

Renpenning, J., Rapp, I., Nijenhuis, I., 2015. Substrate hydrophobicity and cell composition influence the extent of rate limitation and masking of isotope fractionation during microbial reductive dehalogenation of chlorinated ethenes. Environ. Sci. Technol. 49, 4293–4301. https://doi.org/10.1021/es506108j

Rodríguez-Fernández, D., Torrentó, C., Guivernau, M., Viñas, M., Hunkeler, D., Soler, A., Domènech, C., Rosell, M., 2018a. Vitamin B 12 effects on chlorinated methanes-degrading microcosms: Dual isotope and metabolically active microbial populations assessment. Sci. Total Environ. 621, 1615–1625. https://doi.org/10.1016/j.scitotenv.2017.10.067

Rodríguez-Fernández, D., Torrentó, C., Palau, J., Marchesi, M., Soler, A., Hunkeler, D., Domènech, C., Rosell, M., 2018b. Unravelling long-term source removal effects and chlorinated methanes natural attenuation processes by C and Cl stable isotopic patterns at a complex field site. Sci. Total Environ. 645, 286–296. https://doi.org/10.1016/j.scitotenv.2018.07.130

Sherwood Lollar, Barbara, Hirschorn, S., Mundle, S.O.C., Grostern, A., Edwards, E., Lacrampe-Couloume, G., Sherwood Lollar, B., 2010. Insights into enzyme kinetics of chloroethane biodegradation using compound specific stable isotopes. Environ. Sci. Technol. 44, 7498–7503. https://doi.org/10.1021/es101330r

Shouakar-Stash, O., Drimmie, R.J., Zhang, M., Frape, S.K., 2006. Compound-specific chlorine isotope ratios of TCE, PCE and DCE isomers by direct injection using CF-IRMS. Appl. Geochemistry 21, 766–781. https://doi.org/10.1016/j.apgeochem.2006.02.006

Shouakar-Stash, O., Frape, S.K., Drimmie, R.J., 2003. Stable hydrogen, carbon and chlorine isotope measurements of selected chlorinated organic solvents. J. Contam. Hydrol. 60, 211–228. https://doi.org/10.1016/S0169-7722(02)00085-2

Tang, S., Edwards, E., 2013. Identification of Dehalobacter reductive dehalogenases that catalyse dechlorination of chloroform, 1,1,1- trichloroethane and 1,1-dichloroethane. Philos. Trans. R. Soc. B Biol. Sci. 368. https://doi.org/10.1098/rstb.2012.0318

Thullner, M., Kampara, M., Richnow, H.H., Harms, H., Wick, L.Y., 2008. Impact of bioavailability restrictions on microbially induced stable isotope fractionation. 1. Theoretical calculation. Environ. Sci. Technol. 42, 6544–6551. https://doi.org/10.1021/es702782c

Torgonskaya, M.L., Zyakun, A.M., Trotsenko, Y.A., Laurinavichius, K.S., Kümmel, S., Vuilleumier, S., Richnow, H.H., 2019. Individual stages of bacterial dichloromethane degradation mapped by carbon and chlorine stable isotope analysis. J. Environ. Sci. 78, 147–160. https://doi.org/10.1016/j.jes.2018.09.008

Torrentó, C., Palau, J., Rodríguez-Fernández, D., Heckel, B., Meyer, A., Domènech, C., Rosell, M., Soler, A., Elsner, M., Hunkeler, D., 2017. Carbon and chlorine isotope fractionation patterns associated with different engineered chloroform transformation reactions. Environ. Sci. Technol. 51, 6174–6184. https://doi.org/10.1021/acs.est.7b00679

Trueba-Santiso, A., Parladé, E., Rosell, M., Lliros, M., Mortan, S.H., Martínez-Alonso, M., Gaju, N., Martín-González, L., Vicent, T., Marco-Urrea, E., 2017. Molecular and carbon isotopic characterization of an anaerobic stable enrichment culture containing Dehalobacterium sp. during dichloromethane fermentation. Sci. Total Environ. 581–582, 640–648. https://doi.org/10.1016/j.scitotenv.2016.12.174

van Warmerdam, E.M.M., Frape, S.K.K., Aravena, R., Drimmie, R.J., Flatt, H., Cherry, J.A. a, 1995. Stable chlorine and carbon isotope measurement of selected chlorinated organic solvents. Appl. Geochem. 10, 547–552. https://doi.org/10.1016/0883-2927(95)00025-9

Wanner, P., Parker, B.L., Chapman, S.W., Lima, G., Gilmore, A., Mack, E.E., Aravena, R., 2018. Identification of degradation pathways of chlorohydrocarbons in saturated low-permeability sediments using compound-specific isotope analysis. Environ. Sci. Technol. 52, 7296–7306. https://doi.org/10.1021/acs.est.8b01173

Weathers, L.J., Parkin, G.F., 2000. Toxicity of chloroform biotransformation to methanogenic bacteria. Environ. Sci. Technol. 34, 2764–2767. https://doi.org/10.1021/es990948x

Wei, K., Grostern, A., Chan, W.W.M., Richardson, R.E., Edwards, E.A., 2016. Electron acceptor interactions between organohalide-respiring bacteria: cross-feeding, competition, and inhibition, in: L. Adrian and F.E. Löffler (Ed) Organohalide-Respiring Bacteria. Springer Berlin Heidelberg, Berlin, Heidelberg, pp. 283–308. https://doi.org/10.1007/978-3-662-49875-0_13

Wong, Y.K., Holland, S.I., Ertan, H., Manefield, M.J., Lee, M., 2016. Isolation and characterization of Dehalobacter sp. strain UNSWDHB capable of chloroform and chlorinated ethane respiration. Environ. Microbiol. 18, 3092–3105. https://doi.org/10.1111/1462-2920.13287

FIGURE AND CAPTIONS

Figure 1. (A) Concentrations (in mg/L) and δ13C and δ37Cl compositions (in ‰) of CF, DCM, TCE and MCB from Site 1. Pie charts show the molar distribution of contaminants at each well. Detailed concentrations (in µM) and isotopic compositions are available in SI sections G and H, respectively. “n.d.” means “could not be determined based on their low concentration”; “n.a.” means “not analysed”. (B) Dual C–Cl isotopic assessment for DCM and CF field data from Site 1. Both CF0 and the range for DCMini are represented. Green and orange dashed rectangles depict the δ13C and δ37Cl ranges of commercial DCM and CF solvents, respectively (see Table 2 for details).

Figure 2. (A) Concentrations (in mg/L) and δ13C and δ37Cl compositions (in ‰) of CF, DCM, TCE and MCB from Site 2. Pie charts show the molar distribution of contaminants at each well. Detailed concentrations (in µM) and isotopic compositions are available in SI sections G and H, respectively. “n.d.” means “could not be determined based on their low concentration”; “n.a.” means “not analysed”. (B) Dual C–Cl isotopic assessment for DCM and CF field data from Site 2. Both CF0 and the range for DCMini are represented. Green and orange dashed rectangles depict the δ13C and δ37Cl ranges of commercial DCM and CF solvents, respectively (see Table 2 for details).

Figure 3. Double logarithmic Rayleigh plot for chlorine isotope data (A) and dual C–Cl isotope plot (B) for the anaerobic degradation of DCM by the Dhb containing culture investigated in this study. Solid black lines for the Dhb culture in each panel depict the corresponding linear regression and dashed lines represent the associated 95% CI. In panel B, the trend lines reported for DCM degradation by Hyphomicrobium strain MC8b (Heraty et al., 1999), Dhb f. and D. elyunquensis (Chen et al., 2018) are shown for comparison. Data points show the error bars from duplicate measurements.

TABLES

Table 1. εC, εCl and ΛC/Cl of DCM and CF degradation by several mechanismsa.

| DCM | εC (‰) | εCl (‰) | ΛC/Cl | Reference |
| --- | --- | --- | --- | --- |
| Mixed culture containing Dehalobacterium sp. (Dhb) | -31 ± 3* | -5.2 ± 0.6 | +5.9 ± 0.3 | This study and re-calculated from Trueba-Santiso et al. (2017)* |
| Dehalobacterium formicoaceticum (Dhb f.) | -42.4 ± 0.7 | -5.3 ± 0.1 | +7.89 ± 0.12 | Chen et al. (2018) |
| Consortium RM harboring Ca. Dichloromethanomonas elyunquensis | -18.3 ± 0.2 | -5.2 ± 0.1 | +3.40 ± 0.03 | Chen et al. (2018) |
| Mixed culture containing Dehalobacter sp. (DCMD) | -16 ± 2 | n.a.b | n.a. | Lee et al. (2015) |
| Hyphomicrobium sp. strain MC8b | -42.4c,d | -3.8c,d | +11.2c,d | Heraty et al. (1999) |
| Hyphomicrobium strains, Methylobacterium, | -41.2 to -66.3d | n.a. | n.a. | Nikolausz et al. (2006) |
| Methylopila, Methylophilus, and Methylorhabdus strains | -45.8 to -61.0e |  |  | Nikolausz et al. (2006) |
| Hyphomicrobium strains | -22 to -46d | n.a. | n.a. | Hermon et al. (2018) |
|  | -26 ± 5e |  |  | Hermon et al. (2018) |
| Methylobacterium extorquens DM4 (average between high and low density cell suspensions) | -66.6d,f | -7.0d,f | +9.5d,f | Torgonskaya et al. (2019) |
| DM4 DCM dehalogenase (average between high and low activity) | -48.4d,f | -5.7d,f | +8.5d,f | Torgonskaya et al. (2019) |
| DM11 DCM dehalogenase (average between high and low activity) | -45.4d,f | -5.6d,f | +8.1d,f | Torgonskaya et al. (2019) |
| CF | εC (‰) | εCl (‰) | ΛC/Cl | Reference |
| Oxidation | -8 ± 1 | -0.44 ± 0.06 | +17 ± 2 | Torrentó et al. (2017) |
| Alkaline hydrolysis | -57 ± 5 | -4.4 ± 0.4 | +13.0 ± 0.8 | Torrentó et al. (2017) |
| Hydrogenolysis plus reductive elimination with Fe (0) | -33 ± 11 | -3 ± 1 | +8 ± 2 | Torrentó et al. (2017) |
| Biodegradation with vitamin B12 | -14 ± 4 | -2.4 ± 0.4 | +7 ± 1 | Rodríguez-Fernández et al. (2018a) |
| Outer-sphere single electron transfer (OS-SET) | -17.7 ± 0.8 | -2.6 ± 0.2 | +6.7 ± 0.4 | Heckel et al. (2017a) |
| Dehalobacter strain CF | -28 ± 2 | -4.2 ± 0.2 | +6.6 ± 0.1 | Heckel et al. (2019) |
| Dehalobacter strain UNSWDHB | -3.1 ± 0.5 | +2.5 ± 0.3 | -1.2 ± 0.2 | Heckel et al. (2019) |
| Mixed culture containing Dehalobacter strain UNSWDHB | -4.3 ± 0.5 | n.a. | n.a. | Lee et al. (2015) |
| aUncertainties of ε and Λ values correspond to the 95% confidence interval (CI). bn.a., values were not analysed. cΛC/Cl values were calculated based on reported εC and εCl data by the referenced authors. dValues were measured under oxic conditions. eValues were measured under nitrate-reducing conditions. fεC and εCl values were calculated here based on reported αC and αCl data (εC,Cl = 1/αC,Cl – 1) · 1000), and the ΛC/Cl values from the here estimated εC and εCl values (ΛC/Cl ~ εC/εCl). |  |  |  |  |  |

Table 2. δ13C and δ37Cl (in ‰) of commercial pure-phase CF, DCM, TCE and MCB.

| DCM | δ13C (‰/VPDB) | δ37Cl (‰/SMOC) | Reference |
| --- | --- | --- | --- |
| Fisher | -41.71 ± 0.08 | -2.7 ± 0.1 | This study |
| Sigma Aldrich | - | +0.75 ± 0.05 | This study |
| SDS | -35.4 ± 0.2 | +1.27 ± 0.01 | This study |
| Merck | -39.0 ± 0.1 | - | This study |
| unknown | -39.78 ± 0.08 | - | This study |
| unknown | - | +1.3 ± 0.2 | This study |
| unknown | - | +2.1 ± 0.3 | This study |
| unknown | -31.5 ± 0.3 | +2.13 ± 0.03 | Jendrzejewski et al. (2001) |
| unknown | -31.8 ± 0.5 | +2.3 ± 0.2 | Jendrzejewski et al. (2001) |
| unknown | -34.19 ± 0.02 | +1.555 ± 0.005 | Holt et al. (1997) |
| unknown | -40.4 ± 0.5 | - | Holt et al. (1997) |
| CF | δ13C (‰/VPDB) | δ37Cl (‰/SMOC) | Reference |
| Mercka | -48.93 ± 0.08 | - | This study |
| Mercka | -52.7 ± 0.1 | - | This study |
| Sigma Aldrich | - | -2.6 ± 0.1 | This study |
| Alfa | -47.96 ± 0.06 | -5.9 ± 0.2 | This study |
| Alfa | -47.88 ± 0.08 | -5.4 ± 0.3 | Breider (2013) |
| unknown | -43.21 ± 0.04 | -1.52 ± 0.01 | Holt et al. (1997) |
| Fluka | -48.7 ± 0.1 | -3.0 ± 0.2 | Rodríguez-Fernández et al. (2018b), Heckel et al. (2017b) |
| Acros | -49.76 ± 0.08 | - | Rodríguez-Fernández et al. (2018b) |
| unknown | -51.7 ± 0.4 | +0.32 ± 0.08 | Jendrzejewski et al. (2001) |
| Fisher | -53.23 ± 0.09 | - | Rodríguez-Fernández et al. (2018b) |
| Sigma Aldrich | -63.6 ± 0.1 | - | Breider (2013) |
| TCE | δ13C (‰/VPDB) | δ37Cl (‰/SMOC) | Reference |
| unknown | -25.40 ± 0.03 | - | This study |
| unknown | -27.18 ± 0.01 | -1.42 ± 0.10 | Holt et al. (1997) |
| PPG | -27.37 ± 0.09 | -2.8 ± 0.1 | Shouakar-Stash et al. (2003) |
| unknown | -27.90 ± 0.08 | +2.0 ± 0.1 | Jendrzejewski et al. (2001) |
| StanChem | -29.1 ± 0.1 | -3.19 ± 0.07 | Shouakar-Stash et al. (2003) |
| ICI | -31.01 ± 0.09 | +2.71 ± 0.08 | Shouakar-Stash et al. (2003) |
| Dow | -31.57 ± 0.01 | +3.55 ± 0.05 | Shouakar-Stash et al. (2003) |
| Dow | -31.90 ± 0.05 | +4.1 ± 0.3 | van Warmerdam et al. (1995) |
| Sigma Aldrich | -33.49 ± 0.08 | +3.8 ± 0.1 | Jendrzejewski et al. (2001) |
| MCB | δ13C (‰/VPDB) | δ37Cl (‰/SMOC) | Reference |
| Sigma Aldrich | -27.09 ± 0.07 | - | This study |
| Sigma Aldrich | -25.6b | - | Liang et al. (2011) |
| aSolvent with the same reference number but from different batches. bδ13C (‰) value for MCB was obtained from a microcosm experiment where changes in δ13C by biodegradation of MCB were measured. The initial δ13C is not considered to be affected by degradation and, therefore, can be assumed as representative of the commercial MCB. VPDB: Vienna Pee Dee Belemnite, SMOC: Standard Mean Ocean Chloride. |  |  |  |